#include <SoftwareSerial.h>

#include <AltSoftSerial.h>

#include <TinyGPS++.h>

#include <math.h>

#include<Wire.h>

//--------------------------------------------------------------

//emergency phone number with country code

const String EMERGENCY_PHONE = "+916361064355";

//--------------------------------------------------------------

//GSM Module RX pin to Arduino 3

//GSM Module TX pin to Arduino 2

#define rxPin 2

#define txPin 3

SoftwareSerial sim800(rxPin,txPin);

//--------------------------------------------------------------

//GPS Module RX pin to Arduino 9

//GPS Module TX pin to Arduino 8

AltSoftSerial neogps;

TinyGPSPlus gps;

//--------------------------------------------------------------

String sms_status,sender_number,received_date,msg;

String latitude, longitude;

//--------------------------------------------------------------

#define BUZZER 12

#define BUTTON 4

//--------------------------------------------------------------

byte updateflag;

boolean impact_detected = false;

//Used to run impact routine every 2mS.

unsigned long time1;

unsigned long impact_time;

unsigned long alert_delay = 30000; //30 seconds

//--------------------------------------------------------------

/*****************************************************************************************

* setup() function

*****************************************************************************************/

void setup()

{

Serial.begin(9600);  //Serial.println("Arduino serial initialize");

sim800.begin(9600); //Serial.println("SIM800L serial initialize");

neogps.begin(9600);//Serial.println("NEO6M serial initialize");

pinMode(BUZZER, OUTPUT);

pinMode(BUTTON, INPUT_PULLUP);

//--------------------------------------------------------------

sms_status = "";

sender_number="";

received_date="";

msg="";

//--------------------------------------------------------------

sim800.println("AT"); //Check GSM Module

delay(1000);

//SendAT("AT", "OK", 2000); //Check GSM Module

sim800.println("ATE1"); //Echo ON

delay(1000);

//SendAT("ATE1", "OK", 2000); //Echo ON

sim800.println("AT+CPIN?"); //Check SIM ready

delay(1000);

//SendAT("AT+CPIN?", "READY", 2000); //Check SIM ready

sim800.println("AT+CMGF=1"); //SMS text mode

delay(1000);

//SendAT("AT+CMGF=1", "OK", 2000); //SMS text mode

sim800.println("AT+CNMI=1,1,0,0,0"); /// Decides how newly arrived SMS should be handled

delay(1000);

//SendAT("AT+CNMI=1,1,0,0,0", "OK", 2000); //set sms received format

//AT +CNMI = 2,1,0,0,0 - AT +CNMI = 2,2,0,0,0 (both are same)

//--------------------------------------------------------------

time1 = micros();

//Serial.print("time1 = "); Serial.println(time1);

//--------------------------------------------------------------

}

/*****************************************************************************************

* loop() function

*****************************************************************************************/

void loop()

{

if(digitalRead(BUTTON) == LOW)

{

updateflag=0;

Serial.println("Impact detected!!");

getGps();

digitalWrite(BUZZER, HIGH);

delay(1000);

sendAlert();

}

//--------------------------------------------------------------

while(sim800.available()){

parseData(sim800.readString());

}

//--------------------------------------------------------------

while(Serial.available())  {

sim800.println(Serial.readString());

}

//--------------------------------------------------------------

}

/*****************************************************************************************

* parseData() function

*****************************************************************************************/

void parseData(String buff)

{

Serial.println(buff);

unsigned int len, index;

//--------------------------------------------------------------

//Remove sent "AT Command" from the response string.

index = buff.indexOf("\r");

buff.remove(0, index+2);

buff.trim();

//NNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNN

if(buff != "OK")

{

//--------------------------------------------------------------

index = buff.indexOf(":");

String cmd = buff.substring(0, index);

cmd.trim();

buff.remove(0, index+2);

//Serial.println(buff);

//--------------------------------------------------------------

if(cmd == "+CMTI")

{

//get newly arrived memory location and store it in temp

//temp = 4

index = buff.indexOf(",");

String temp = buff.substring(index+1, buff.length());

temp = "AT+CMGR=" + temp + "\r";

//AT+CMGR=4 i.e. get message stored at memory location 4

sim800.println(temp);

}

//--------------------------------------------------------------

else if(cmd == "+CMGR")

{

//extractSms(buff);

//Serial.println(buff.indexOf(EMERGENCY_PHONE));

if(buff.indexOf(EMERGENCY_PHONE) > 1)

{

buff.toLowerCase();

//Serial.println(buff.indexOf("get gps"));

if(buff.indexOf("get gps") > 1)

{

getGps();

String sms_data;

sms_data = "GPS Location Data\r";

sms_data += "http://maps.google.com/maps?q=loc:";

sms_data += latitude + "," + longitude;

sendSms(sms_data);

}

}

}

//--------------------------------------------------------------

}

else

{

//The result of AT Command is "OK"

}

//NNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNNN

}

/*****************************************************************************************

* getGps() Function

*****************************************************************************************/

void getGps()

{

// Can take up to 60 seconds

boolean newData = false;

for (unsigned long start = millis(); millis() - start < 2000;)

{

while (neogps.available())

{

if (gps.encode(neogps.read()))

{

newData = true;

break;

}

}

}

if (newData) //If newData is true

{

latitude = String(gps.location.lat(), 6);

longitude = String(gps.location.lng(), 6);

newData = false;

}

else

{

Serial.println("No GPS data is available");

latitude = "";

longitude = "";

}

Serial.print("Latitude= "); Serial.println(latitude);

Serial.print("Lngitude= "); Serial.println(longitude);

}

/*****************************************************************************************

* sendAlert() function

*****************************************************************************************/

void sendAlert()

{

String sms_data;

sms_data = "I'm in Danger Please Help Me,\r";

sms_data += "http://maps.google.com/maps?q=loc:";

sms_data += latitude + "," + longitude;

sendSms(sms_data);

}

/*****************************************************************************************

* sendSms() function

*****************************************************************************************/

void sendSms(String text)

{

//return;

sim800.print("AT+CMGF=1\r");

delay(1000);

sim800.print("AT+CMGS=\""+EMERGENCY_PHONE+"\"\r");

delay(1000);

sim800.print(text);

delay(100);

sim800.write(0x1A); //ascii code for ctrl-26 //sim800.println((char)26); //ascii code for ctrl-26

delay(1000);

Serial.println("SMS Sent Successfully.");

}

/*****************************************************************************************

* SendAT() function

*****************************************************************************************/

boolean SendAT(String at_command, String expected_answer, unsigned int timeout)

{

uint8_t x=0;

boolean answer=0;

String response;

unsigned long previous;

//Clean the input buffer

while( sim800.available() > 0) sim800.read();

sim800.println(at_command);

x = 0;

previous = millis();

//this loop waits for the answer with time out

do

{

//if there are data in the UART input buffer, reads it and checks for the asnwer

if(sim800.available() != 0)

{

response += sim800.read();

x++;

// check if the desired answer (OK) is in the response of the module

if(response.indexOf(expected_answer) > 0)

{

answer = 1;

break;

}

}

}

while((answer == 0) && ((millis() - previous) < timeout));

Serial.println(response);

return answer;

}